Instructional Guide for Flashing Firmware via USB Cradle

IMPORTANT NOTE: Flashing a Firmware via USB cradle only works in a Windows platform because the hardware driver is available for Windows only.

Here are the things you need before flashing the latest Firmware on your Kreyos Meteor via USB cradle:

- Kreyos Meteor

- Kreyos Meteor charger (USB cradle)

- ‘KreyosFirmware’ folder

- This folder contains the following files:

  - BSL executable file (BSL.exe)

  - run batch file (run.bat)

  - firmware binary file version 1.0.0.1071 (firmware.bin)

  - Commands: (prog, reset)

- ‘CP210x_VCP_Windows’ folder (Hardware driver)

There are 2 commands that you can input in Command Prompt:

- bsl reset

  - Initial log checking on the watch

- bsl prog/run

  - flush the new firmware to watch

Here are the steps for flashing the firmware to the watch via the USB cradle:

- Install the driver

  - To do this, go to the ‘CP210x_VCP_Windows’ folder

  - Click on either ‘CP210xVCPInstaller_x64’ or ‘CP210xVCPInstaller_x86’ depending on your OS system type. to install the hardware driver

  - IMPORTANT NOTE: ‘CP210xVCPInstaller_x64’ is for 64-bit OS while ‘CP210xVCPInstaller_x86’ is for 32-bit OS.

- Plug the watch to your computer

  - Make sure the “Battery is charging.” screen is showing on the watch

- Locate the 'KreyosFirmware' folder

  - To do this, open the Command Prompt

  - Locate the 'KreyosFirmware' folder directory

  - IMPORTANT NOTE: Copy the ‘KreyosFirmware’ folder to drive C: for easier access.

- Run the command 'bsl reset' to check if watch is connected

  - After locating the ‘KreyosFirmware’ folder on the Command Prompt, enter “bsl reset” to initialize checking on the watch. The Command Prompt should display the following:

  - IMPORTANT NOTE: Press Ctrl+C to be able to run a new command

- Run the command ‘run' or ‘bsl prog’ to flush the firmware.